4-bit full adder

Digital Logic Design Project

22/05/2023

Timeline:

Yousra Amr Ali

2022/05494

Mariam Jan George

2022/06056

Members:

Dr. Khaled Hussein

Supervisor:

Contents

Idea	3

Objective	4

K-maps	4

Boolean Function	5

Code of Truth Table	6

Truth Table	8

Simulator	9

Simulator with IC	10

Hardware Data Sheet	11

Components of Hardware	12

4 – Bit full adder

Idea

- A four bit full Adders is a Logical Circuit that takes Carry and two expressions with four bits as its inputs (A and B) and a carry-in bit (C-in), and as a result shows the Four bits and two outputs: the sum of the two numbers (S) and a carry-out bit (C-out)." The Circuit of Four bit Full Adder consists of the XOR Gate, AND Gate and OR Gate. We have learnt about them in detail.

- 4-bit full adder is used to minimize the total delay of the adder, the area used to implement the adder, and its average power consumption.

Objective

- 4-bit full adder performs the function of 4-bit addition that gives a sum and two bits of carry as output.

K-maps & Full adder Diagram

Boolean Function

S0 = A0 ⊕ B0 ⊕ Cin

Cout0 = A Cin + A B + B Cin

S1 = A1 ⊕ B1 ⊕ Cout0

Cout1 = A Cin + A B + B Cin

S2 = A2 ⊕ B2 ⊕ Cout1

Cout2 = A Cin + A B + B Cin

S3 = A3 ⊕ B3 ⊕ Cout2

Cout3 = A Cin + A B + B Cin

Code of Truth Table

#include <iostream>

#include <iomanip>

using namespace std;

void generateTruthTable() {

int a[4], b[4], carryIn, carryOut, sum[4];

cout << setw(4) << "A3" << setw(4) << "A2" << setw(4) << "A1" << setw(4) << "A0"

<< setw(4) << "B3" << setw(4) << "B2" << setw(4) << "B1" << setw(4) << "B0"

<< setw(8) << "CarryIn"

<< setw(10) << "CarryOut"

<< setw(8) << "Sum3" << setw(8) << "Sum2" << setw(8) << "Sum1" << setw(8) << "Sum0" << endl;

for (int i = 0; i < 16; ++i) {

a[3] = (i & 8) >> 3;

a[2] = (i & 4) >> 2;

a[1] = (i & 2) >> 1;

a[0] = (i & 1);

for (int j = 0; j < 16; ++j) {

b[3] = (j & 8) >> 3;

b[2] = (j & 4) >> 2;

b[1] = (j & 2) >> 1;

b[0] = (j & 1);

carryIn = 0;

carryOut = 0;

for (int k = 0; k < 4; ++k) {

sum[k] = a[k] ^ b[k] ^ carryIn;

carryOut = (a[k] & b[k]) | ((a[k] ^ b[k]) & carryIn);

carryIn = carryOut;

}

cout << setw(4) << a[3] << setw(4) << a[2] << setw(4) << a[1] << setw(4) << a[0]

<< setw(4) << b[3] << setw(4) << b[2] << setw(4) << b[1] << setw(4) << b[0]

<< setw(8) << carryIn

<< setw(10) << carryOut

<< setw(8) << sum[3] << setw(8) << sum[2] << setw(8) << sum[1] << setw(8) << sum[0] << endl;

}

}

}

int main() {

generateTruthTable();

}

Truth Table

Code of Truth Table

de of Truth Table

A3 A2A1 A0 B3 B2  B1  B0 CIn  COut    S3    S2    S1    S0

0   0   0   0   0   0   0   0       0         0       0       0       0       0

0   0   0   0   0   0   0   1       0         0       0       0       0       1

0   0   0   0   0   0   1   0       0         0       0       0       1       0

0   0   0   0   0   0   1   1       0         0       0       0       1       1

0   0   0   0   0   1   0   0       0         0       0       1       0       0

0   0   0   0   0   1   0   1       0         0       0       1       0       1

0   0   0   0   0   1   1   0       0         0       0       1       1       0

0   0   0   0   0   1   1   1       0         0       0       1       1       1

0   0   0   0   1   0   0   0       0         0       1       0       0       0

0   0   0   0   1   0   0   1       0         0       1       0       0       1

0   0   0   0   1   0   1   0       0         0       1       0       1       0

0   0   0   0   1   0   1   1       0         0       1       0       1       1

0   0   0   0   1   1   0   0       0         0       1       1       0       0

0   0   0   0   1   1   0   1       0         0       1       1       0       1

0   0   0   0   1   1   1   0       0         0       1       1       1       0

0   0   0   0   1   1   1   1       0         0       1       1       1       1

0   0   0   1   0   0   0   0       0         0       0       0       0       1

0   0   0   1   0   0   0   1       0         0       0       0       1       0

0   0   0   1   0   0   1   0       0         0       0       0       1       1

0   0   0   1   0   0   1   1       0         0       0       1       0       0

0   0   0   1   0   1   0   0       0         0       0       1       0       1

0   0   0   1   0   1   0   1       0         0       0       1       1       0

0   0   0   1   0   1   1   0       0         0       0       1       1       1

0   0   0   1   0   1   1   1       0         0       1       0       0       0

0   0   0   1   1   0   0   0       0         0       1       0       0       1

0   0   0   1   1   0   0   1       0         0       1       0       1       0

0   0   0   1   1   0   1   0       0         0       1       0       1       1

0   0   0   1   1   0   1   1       0         0       1       1       0       0

0   0   0   1   1   1   0   0       0         0       1       1       0       1

0   0   0   1   1   1   0   1       0         0       1       1       1       0

0   0   0   1   1   1   1   0       0         0       1       1       1       1

0   0   0   1   1   1   1   1       1         1       0       0       0       0

0   0   1   0   0   0   0   0       0         0       0       0       1       0

0   0   1   0   0   0   0   1       0         0       0       0       1       1

0   0   1   0   0   0   1   0       0         0       0       1       0       0

0   0   1   0   0   0   1   1       0         0       0       1       0       1

0   0   1   0   0   1   0   0       0         0       0       1       1       0

0   0   1   0   0   1   0   1       0         0       0       1       1       1

0   0   1   0   0   1   1   0       0         0       1       0       0       0

0   0   1   0   0   1   1   1       0         0       1       0       0       1

0   0   1   0   1   0   0   0       0         0       1       0       1       0

0   0   1   0   1   0   0   1       0         0       1       0       1       1

0   0   1   0   1   0   1   0       0         0       1       1       0       0

0   0   1   0   1   0   1   1       0         0       1       1       0       1

0   0   1   0   1   1   0   0       0         0       1       1       1       0

0   0   1   0   1   1   0   1       0         0       1       1       1       1

0   0   1   0   1   1   1   0       1         1       0       0       0       0

0   0   1   0   1   1   1   1       1         1       0       0       0       1

0   0   1   1   0   0   0   0       0         0       0       0       1       1

0   0   1   1   0   0   0   1       0         0       0       1       0       0

0   0   1   1   0   0   1   0       0         0       0       1       0       1

0   0   1   1   0   0   1   1       0         0       0       1       1       0

0   0   1   1   0   1   0   0       0         0       0       1       1       1

0   0   1   1   0   1   0   1       0         0       1       0       0       0

0   0   1   1   0   1   1   0       0         0       1       0       0       1

0   0   1   1   0   1   1   1       0         0       1       0       1       0

0   0   1   1   1   0   0   0       0         0       1       0       1       1

0   0   1   1   1   0   0   1       0         0       1       1       0       0

0   0   1   1   1   0   1   0       0         0       1       1       0       1

0   0   1   1   1   0   1   1       0         0       1       1       1       0

0   0   1   1   1   1   0   0       0         0       1       1       1       1

0   0   1   1   1   1   0   1       1         1       0       0       0       0

0   0   1   1   1   1   1   0       1         1       0       0       0       1

0   0   1   1   1   1   1   1       1         1       0       0       1       0

0   1   0   0   0   0   0   0       0         0       0       1       0       0

0   1   0   0   0   0   0   1       0         0       0       1       0       1

0   1   0   0   0   0   1   0       0         0       0       1       1       0

0   1   0   0   0   0   1   1       0         0       0       1       1       1

0   1   0   0   0   1   0   0       0         0       1       0       0       0

0   1   0   0   0   1   0   1       0         0       1       0       0       1

0   1   0   0   0   1   1   0       0         0       1       0       1       0

0   1   0   0   0   1   1   1       0         0       1       0       1       1

0   1   0   0   1   0   0   0       0         0       1       1       0       0

0   1   0   0   1   0   0   1       0         0       1       1       0       1

0   1   0   0   1   0   1   0       0         0       1       1       1       0

0   1   0   0   1   0   1   1       0         0       1       1       1       1

0   1   0   0   1   1   0   0       1         1       0       0       0       0

0   1   0   0   1   1   0   1       1         1       0       0       0       1

0   1   0   0   1   1   1   0       1         1       0       0       1       0

0   1   0   0   1   1   1   1       1         1       0       0       1       1

0   1   0   1   0   0   0   0       0         0       0       1       0       1

0   1   0   1   0   0   0   1       0         0       0       1       1       0

0   1   0   1   0   0   1   0       0         0       0       1       1       1

0   1   0   1   0   0   1   1       0         0       1       0       0       0

0   1   0   1   0   1   0   0       0         0       1       0       0       1

0   1   0   1   0   1   0   1       0         0       1       0       1       0

0   1   0   1   0   1   1   0       0         0       1       0       1       1

0   1   0   1   0   1   1   1       0         0       1       1       0       0

0   1   0   1   1   0   0   0       0         0       1       1       0       1

0   1   0   1   1   0   0   1       0         0       1       1       1       0

0   1   0   1   1   0   1   0       0         0       1       1       1       1

0   1   0   1   1   0   1   1       1         1       0       0       0       0

0   1   0   1   1   1   0   0       1         1       0       0       0       1

0   1   0   1   1   1   0   1       1         1       0       0       1       0

0   1   0   1   1   1   1   0       1         1       0       0       1       1

0   1   0   1   1   1   1   1       1         1       0       1       0       0

0   1   1   0   0   0   0   0       0         0       0       1       1       0

0   1   1   0   0   0   0   1       0         0       0       1       1       1

0   1   1   0   0   0   1   0       0         0       1       0       0       0

0   1   1   0   0   0   1   1       0         0       1       0       0       1

0   1   1   0   0   1   0   0       0         0       1       0       1       0

0   1   1   0   0   1   0   1       0         0       1       0       1       1

0   1   1   0   0   1   1   0       0         0       1       1       0       0

0   1   1   0   0   1   1   1       0         0       1       1       0       1

0   1   1   0   1   0   0   0       0         0       1       1       1       0

0   1   1   0   1   0   0   1       0         0       1       1       1       1

0   1   1   0   1   0   1   0       1         1       0       0       0       0

0   1   1   0   1   0   1   1       1         1       0       0       0       1

0   1   1   0   1   1   0   0       1         1       0       0       1       0

0   1   1   0   1   1   0   1       1         1       0       0       1       1

0   1   1   0   1   1   1   0       1         1       0       1       0       0

0   1   1   0   1   1   1   1       1         1       0       1       0       1

0   1   1   1   0   0   0   0       0         0       0       1       1       1

0   1   1   1   0   0   0   1       0         0       1       0       0       0

0   1   1   1   0   0   1   0       0         0       1       0       0       1

0   1   1   1   0   0   1   1       0         0       1       0       1       0

0   1   1   1   0   1   0   0       0         0       1       0       1       1

0   1   1   1   0   1   0   1       0         0       1       1       0       0

0   1   1   1   0   1   1   0       0         0       1       1       0       1

0   1   1   1   0   1   1   1       0         0       1       1       1       0

0   1   1   1   1   0   0   0       0         0       1       1       1       1

0   1   1   1   1   0   0   1       1         1       0       0       0       0

0   1   1   1   1   0   1   0       1         1       0       0       0       1

0   1   1   1   1   0   1   1       1         1       0       0       1       0

0   1   1   1   1   1   0   0       1         1       0       0       1       1

0   1   1   1   1   1   0   1       1         1       0       1       0       0

0   1   1   1   1   1   1   0       1         1       0       1       0       1

0   1   1   1   1   1   1   1       1         1       0       1       1       0

1   0   0   0   0   0   0   0       0         0       1       0       0       0

1   0   0   0   0   0   0   1       0         0       1       0       0       1

1   0   0   0   0   0   1   0       0         0       1       0       1       0

1   0   0   0   0   0   1   1       0         0       1       0       1       1

1   0   0   0   0   1   0   0       0         0       1       1       0       0

1   0   0   0   0   1   0   1       0         0       1       1       0       1

1   0   0   0   0   1   1   0       0         0       1       1       1       0

1   0   0   0   0   1   1   1       0         0       1       1       1       1

1   0   0   0   1   0   0   0       1         1       0       0       0       0

1   0   0   0   1   0   0   1       1         1       0       0       0       1

1   0   0   0   1   0   1   0       1         1       0       0       1       0

1   0   0   0   1   0   1   1       1         1       0       0       1       1

1   0   0   0   1   1   0   0       1         1       0       1       0       0

1   0   0   0   1   1   0   1       1         1       0       1       0       1

1   0   0   0   1   1   1   0       1         1       0       1       1       0

1   0   0   0   1   1   1   1       1         1       0       1       1       1

1   0   0   1   0   0   0   0       0         0       1       0       0       1

1   0   0   1   0   0   0   1       0         0       1       0       1       0

1   0   0   1   0   0   1   0       0         0       1       0       1       1

1   0   0   1   0   0   1   1       0         0       1       1       0       0

1   0   0   1   0   1   0   0       0         0       1       1       0       1

1   0   0   1   0   1   0   1       0         0       1       1       1       0

1   0   0   1   0   1   1   0       0         0       1       1       1       1

1   0   0   1   0   1   1   1       1         1       0       0       0       0

1   0   0   1   1   0   0   0       1         1       0       0       0       1

1   0   0   1   1   0   0   1       1         1       0       0       1       0

1   0   0   1   1   0   1   0       1         1       0       0       1       1

1   0   0   1   1   0   1   1       1         1       0       1       0       0

1   0   0   1   1   1   0   0       1         1       0       1       0       1

1   0   0   1   1   1   0   1       1         1       0       1       1       0

1   0   0   1   1   1   1   0       1         1       0       1       1       1

1   0   0   1   1   1   1   1       1         1       1       0       0       0

1   0   1   0   0   0   0   0       0         0       1       0       1       0

1   0   1   0   0   0   0   1       0         0       1       0       1       1

1   0   1   0   0   0   1   0       0         0       1       1       0       0

1   0   1   0   0   0   1   1       0         0       1       1       0       1

1   0   1   0   0   1   0   0       0         0       1       1       1       0

1   0   1   0   0   1   0   1       0         0       1       1       1       1

1   0   1   0   0   1   1   0       1         1       0       0       0       0

1   0   1   0   0   1   1   1       1         1       0       0       0       1

1   0   1   0   1   0   0   0       1         1       0       0       1       0

1   0   1   0   1   0   0   1       1         1       0       0       1       1

1   0   1   0   1   0   1   0       1         1       0       1       0       0

1   0   1   0   1   0   1   1       1         1       0       1       0       1

1   0   1   0   1   1   0   0       1         1       0       1       1       0

1   0   1   0   1   1   0   1       1         1       0       1       1       1

1   0   1   0   1   1   1   0       1         1       1       0       0       0

1   0   1   0   1   1   1   1       1         1       1       0       0       1

1   0   1   1   0   0   0   0       0         0       1       0       1       1

1   0   1   1   0   0   0   1       0         0       1       1       0       0

1   0   1   1   0   0   1   0       0         0       1       1       0       1

1   0   1   1   0   0   1   1       0         0       1       1       1       0

1   0   1   1   0   1   0   0       0         0       1       1       1       1

1   0   1   1   0   1   0   1       1         1       0       0       0       0

1   0   1   1   0   1   1   0       1         1       0       0       0       1

1   0   1   1   0   1   1   1       1         1       0       0       1       0

1   0   1   1   1   0   0   0       1         1       0       0       1       1

1   0   1   1   1   0   0   1       1         1       0       1       0       0

1   0   1   1   1   0   1   0       1         1       0       1       0       1

1   0   1   1   1   0   1   1       1         1       0       1       1       0

1   0   1   1   1   1   0   0       1         1       0       1       1       1

1   0   1   1   1   1   0   1       1         1       1       0       0       0

1   0   1   1   1   1   1   0       1         1       1       0       0       1

1   0   1   1   1   1   1   1       1         1       1       0       1       0

1   1   0   0   0   0   0   0       0         0       1       1       0       0

1   1   0   0   0   0   0   1       0         0       1       1       0       1

1   1   0   0   0   0   1   0       0         0       1       1       1       0

1   1   0   0   0   0   1   1       0         0       1       1       1       1

1   1   0   0   0   1   0   0       1         1       0       0       0       0

1   1   0   0   0   1   0   1       1         1       0       0       0       1

1   1   0   0   0   1   1   0       1         1       0       0       1       0

1   1   0   0   0   1   1   1       1         1       0       0       1       1

1   1   0   0   1   0   0   0       1         1       0       1       0       0

1   1   0   0   1   0   0   1       1         1       0       1       0       1

1   1   0   0   1   0   1   0       1         1       0       1       1       0

1   1   0   0   1   0   1   1       1         1       0       1       1       1

1   1   0   0   1   1   0   0       1         1       1       0       0       0

1   1   0   0   1   1   0   1       1         1       1       0       0       1

1   1   0   0   1   1   1   0       1         1       1       0       1       0

1   1   0   0   1   1   1   1       1         1       1       0       1       1

1   1   0   1   0   0   0   0       0         0       1       1       0       1

1   1   0   1   0   0   0   1       0         0       1       1       1       0

1   1   0   1   0   0   1   0       0         0       1       1       1       1

1   1   0   1   0   0   1   1       1         1       0       0       0       0

1   1   0   1   0   1   0   0       1         1       0       0       0       1

1   1   0   1   0   1   0   1       1         1       0       0       1       0

1   1   0   1   0   1   1   0       1         1       0       0       1       1

1   1   0   1   0   1   1   1       1         1       0       1       0       0

1   1   0   1   1   0   0   0       1         1       0       1       0       1

1   1   0   1   1   0   0   1       1         1       0       1       1       0

1   1   0   1   1   0   1   0       1         1       0       1       1       1

1   1   0   1   1   0   1   1       1         1       1       0       0       0

1   1   0   1   1   1   0   0       1         1       1       0       0       1

1   1   0   1   1   1   0   1       1         1       1       0       1       0

1   1   0   1   1   1   1   0       1         1       1       0       1       1

1   1   0   1   1   1   1   1       1         1       1       1       0       0

1   1   1   0   0   0   0   0       0         0       1       1       1       0

1   1   1   0   0   0   0   1       0         0       1       1       1       1

1   1   1   0   0   0   1   0       1         1       0       0       0       0

1   1   1   0   0   0   1   1       1         1       0       0       0       1

1   1   1   0   0   1   0   0       1         1       0       0       1       0

1   1   1   0   0   1   0   1       1         1       0       0       1       1

1   1   1   0   0   1   1   0       1         1       0       1       0       0

1   1   1   0   0   1   1   1       1         1       0       1       0       1

1   1   1   0   1   0   0   0       1         1       0       1       1       0

1   1   1   0   1   0   0   1       1         1       0       1       1       1

1   1   1   0   1   0   1   0       1         1       1       0       0       0

1   1   1   0   1   0   1   1       1         1       1       0       0       1

1   1   1   0   1   1   0   0       1         1       1       0       1       0

1   1   1   0   1   1   0   1       1         1       1       0       1       1

1   1   1   0   1   1   1   0       1         1       1       1       0       0

1   1   1   0   1   1   1   1       1         1       1       1       0       1

1   1   1   1   0   0   0   0       0         0       1       1       1       1

1   1   1   1   0   0   0   1       1         1       0       0       0       0

1   1   1   1   0   0   1   0       1         1       0       0       0       1

1   1   1   1   0   0   1   1       1         1       0       0       1       0

1   1   1   1   0   1   0   0       1         1       0       0       1       1

1   1   1   1   0   1   0   1       1         1       0       1       0       0

1   1   1   1   0   1   1   0       1         1       0       1       0       1

1   1   1   1   0   1   1   1       1         1       0       1       1       0

1   1   1   1   1   0   0   0       1         1       0       1       1       1

1   1   1   1   1   0   0   1       1         1       1       0       0       0

1   1   1   1   1   0   1   0       1         1       1       0       0       1

1   1   1   1   1   0   1   1       1         1       1       0       1       0

1   1   1   1   1   1   0   0       1         1       1       0       1       1

1   1   1   1   1   1   0   1       1         1       1       1       0       0

1   1   1   1   1   1   1   0       1         1       1       1       0       1

1   1   1   1   1   1   1   1       1         1       1       1       1       0

Simulator

Circuit’s link:

https://circuitverse.org/simulator/embed/ﬁnalsimulatoryarab

Simulator with IC

https://circuitverse.org/simulator/embed/4-bitfulladder_with_ic

Circuit’s Link:

Hardware Data Sheet

Components of Hardware

- switches 4 bit

- 13 resistor 1k

- IC of 4-bit full adder

- A power supply of 6 volts

- Connection wires

- Breadboard